Sample Submission Guidelines

Use the following guidelines to prepare and ship your samples.  Samples can be submitted in form of live bacterial culture – agar plates, glycerol stock, slants/transportation swabs; in form of bacterial lysate or purified DNA. Some instructions are specific to the format you select as noted below.

Your signed Service Order Form is required to pre-verify and process your samples. Please include the Service Order Form in the package with your submitted samples.

Pure culture:  All samples submitted must be pure single colony cultures to insure accurate analysis.  Contact ID Genomics if you need to submit mixed culture samples.

Shipping temperature: Except for Glycerol Stock, all live samples should be shipped at room temperature and should not be exposed to extreme temperatures to ensure the viability of the samples.

Caution – Excluded microbes: please do not submit live samples requiring BSL-3 or higher level laboratory handling or which can be classified as Category A according to Guidance on regulations for the Transport of Infectious Substances, World Health Organization.

Packaging and Shipping*: The following are recommended packaging and shipping methods for shipping within the USA, but you are responsible to ensure that your packaging and shipping is done in conformance with applicable national and international regulations.  Consult with your in-house shipping and safety staff, your carrier, IATA Infectious Substance Classification & Packing Guidelines, and Guidance on regulations for the Transport of Infectious Substances, World Health Organization for further information.

Shipping address: All packages should be addressed to:

ID Genomics SPC.

3101 Western Ave

Suite 100

Seattle, WA 98121

P. (206)8261910

Weekday deliveries only: We are not able to receive packages on Saturday.  Be sure to time shipping of time sensitive samples to deliver on Monday through Friday.

### 1. Sample preparation

### a. Live samples – Agar plate, agar slant/swab format

Live samples may be submitted as single colony cultures on agar plates, agar slants in tubes or transportation swabs.  If you submit plates do not streak more than 8 strains per 100 mm diameter petri dish and keep each strain within its marked grid (see Labeling section below).  Agar slant tubes and swabs should contain only one sample per tube.  Live samples may require an added fee for Subculturing and DNA Extraction.

### b. Glycerol Stock

Live samples may also be submitted as glycerol stock.  Extract culture grown from a single colony and mix evenly into a minimum 15% glycerol containing solution and place in a freezer compatible sealed tube.  Live samples may require an added fee for Subculturing and DNA Extraction.

### c. Bacterial lysates

Samples may be submitted in form of bacterial lysate. It should be prepared from a single colony culture. Samples may be prepared by the following boiling protocol below.

• Re-suspend bacterial culture in 100 µl of water or 10 mM Tris.  For best results, do not use Tris-EDTA (TE) because EDTA will inhibit the PCR reaction.  Make sure to add enough bacteria so that bacterial solution is not transparent.

• Boil bacterial solution at 95°C for 10 min, spin hard to precipitate all bacterial debris, transfer cleared DNA solution into new tubes.

### c. PCR Ready DNA samples

Samples may be submitted as purified genomic DNA (PCR Ready format) to simplify shipping and save DNA Isolation costs.  Samples must be extracted from a pure culture.  Contact us if you need to submit mixed culture samples.

We accept DNA in water or ph7.5 – 10 mM Tris. EDTA in the DNA sample is not acceptable because it may interfere with PCR and sequencing reactions.

Total DNA amount should be no less than 200 ng. Concentration should be no less than 10ng/µl.

With concentration 10ng/µl we'll need 20µl of DNA. This includes extra volume for repeats of PCR if needed.

Preferred 260/280 ratio is ~1.8

260/230 ratio is 2.0.

### 2. Labeling

### a. Agar plates

(On the bottom of the plate)

• If plating more than one sample on a plate, draw a grid on the back of the plate with indelible ink pen and plate each culture within one grid area, being sure to keep samples well isolated.  Place no more than 8 samples per plate.

• If you submit a mixed colony sample, circle the colony you want analyzed and label it with its Customer Sample ID (within its grid area if gridded).  Customer Sample ID must match the ID entered for each sample on the Service Order Form.

### b. Agar slants/swabs, Glycerol Stock and Bacterial lysates

• Label each tube with the Customer Sample ID corresponding to your Service Order Form.

• Only one bacterial culture per tube.

### c. DNA

• If shipping DNA in tubes or strip wells, label each tube with the Customer Sample ID corresponding to your Service Order Form.

• If shipping DNA in 96-well plates, label the plate clearly with the Quote Number and company initials as listed in the Quote.  Wells should be loaded starting with the left column (A1) and loading top to bottom, then proceeding with the next column to the right (i.e., rows top to bottom, columns left to right). Include information about samples in each well in written format.

• Each well / tube must have DNA from only one sample.

### 3. Recommended packaging*

• Seal each bacterial agar plate with Parafilm.  Make sure tubes are tightly capped and sealed with Parafilm. Strips and 96 well plate with DNA must be capped.  Glycerol Stock samples must be shipped on dry ice per shipping guidelines.

• Enclose a completed and signed copy of Service Order Form in the shipping container.

• We recommend packing according to IATA Packaging Instruction 650.

• Enclose plates and tubes in a primary container.

• Enclose primary container in secondary shipping container.

• Add absorbent and cushioning packing material to protect contents and prevent leakage.

### 4. Shipping

• Ship to the shipping address listed above.  Tracking of your package is recommended.

• Special outer labeling on the package and inclusion of a Shipper’s Declaration of Dangerous Goods may be required, especially for packages containing Category B samples.  See IATA Packaging Instructions 650 for guidelines.

• Be sure to include the Airway Bill on the package and the Service Order Form inside the package.

© ID Genomics SPC.

• 3101 Western Avenue, Suite 100

Seattle, WA 98121

• Phone (206)8261910

• email orders@idgenomics.com